This is additional documentation to describe the changes made

to the Motorola Cross-assemblers download from the Motorola

BB.  Please read the Motorola.DOC file for more

specific instructions on how to use the crossassemblers.

The assemblers were made available from Motorola in C source.

I originally compilied them with Microsoft Quick-C and then with

the Microsoft standard C compilier.  If you want to learn C,

this is a good way!

IMPROVED LISTING FORMAT

The listing format has been extensively modified to:

A) Print a Header with NAMe, sttl and date/time.

B) Print a footer with software ID and page number.

C) Format the listing in an orderly way.

Typical header source

pag

opt nol

NAM ACS MONITOR, REV 5.0

STTL Monitor for Digital Attitude Control Systems

swnam	SW90542 REV B

opt l

Typical listing file

ACS MONITOR, REV 5.0	Motorola Cross Assembler   Thu Sep 07 21:34:25 1989

Monitor for Digital Attitude Control Systems

0365 F1E2 8E E0 00           LOROM    LDX #$E000       Begin acs rom

0366 F1E5 10 8E E7 FF                 LDY #$E7FF       End acs rom

0367 F1E9 20 07                       BRA ROMTS1

0368 F1EB 8E F0 00           HIROM    LDX #$F000       Begin monitor rom

0369 F1EE 10 8E FF FF                 LDY #$FFFF       End monitor rom

0370 F1F2 36 30              ROMTS1   PSHU Y,X         Save addr on ustk

0371 F1F4 30 8D 00 34                 LEAX ROMHDR,PCR  Print header

0372 F1F8 17 02 F1                    LBSR PDATA

0373 F1FB 1F 31                       TFR U,X          Point to begin adr

*************Code removed *****************

SW90542 REV B  PAGE 9

HDRSTG is in the C source file "AS.H" and can be modified

(using AEDIT or any "document" editor) to suit your fancy.

In AS.H

#define HDRSTG	"Motorola Cross Assembler"

can be changed to:

#define HDRSTG	"Your Personal Header"

Please note that the above listing has been compressed somewhat -- by

removing TABs -- to fit on an 80 col screen. The actual generated listing

requires a 132 col printer.

COMMAND LINE OPTIONS

Additional command line options have been added. These are:

g	Display all the code generated by FCC, FCB and FDB.

The default is to display only the first six bytes.

i	Generate the object record in Intel .HEX format rather

than the default Motorola .S19 format. This option was

added because more EPROM burners accept the .HEX format

BATCH FILES

Several useful batch files have been downloaded which should make

assembly easier. As an example:

Asm5.bat   =  AS5 %1.ASM - l >%1.LST

Asm9i.bat  =  AS9 %1.ASM - l i >%1.LST

Place these .BAT files  and the as*.EXE files in your boot-up path.

Typing ASM9I 'filename' will assemble 'filename'.ASM and generate a

'filename'.HEX object file and a 'filename'.LST listing file.

CODE CHECKSUM

A method has been added to imbed in the object code either a zero

or exclusive-OR checksum. This is helpful for memory self-test

techniques. If an 'opt' flag is set, all the object code generated

is checksummed.  The checksum can then be taged to the object code

by reORGing the program counter inside the code and writting the

mnemonic 'CKSUM'. The code fragments below shows the method.

Before any code, set the option flag. i.e.

opt xck		Exclusive OR Checksum

or

opt zck		Zero Checksum

At the end of the code, flag as such

F90F                    ENDROM   EQU *

* INTERRUPT VECTORS

FFF6                             ORG $FFF6

FFF6 F0 F1              FIRQ     FDB FIRQ1

FFF8 F1 70              IRQ      FDB INTRPT

FFFA F5 29                       FDB SWI

FFFC F5 2D                       FDB NMI

FFFE F5 31              RESET    FDB RESTRT

Clear the empty region

06E7                    CLRLTH   EQU FIRQ-ENDROM

F90F                             ORG ENDROM

F90F 00 00 00 00 00 00           FILL 0,CLRLTH (Use FILL $FF for eproms)

FFF5                             ORG FIRQ-1	(A clean spot for the checksum)

FFF5 C6                          CKSUM		(Insert the checksum)

IMPROVED FCC

The FCC handler has been modified to allow *trailing* control characters.

The following will generate correct object code (and wouldn't in the

Motorola version).

MTMS	FCB 10,13

MTMS1	FCC 'MONITOR',10,13,0

Stewart D. (Stu) Lyon

73277,342

19943 Arminta St.

Winnetka, CA 91306

Sept 8,1989