The IBM PC 6800/01/04/05/09/11 cross assemblers

GENERAL

The assemblers are named as*.exe where '*' is  any of 0,1,h1,4,5,9

or 11 depending on which one you're using.  Command line arguments

specify the filenames to assemble.

The assemblers accept options from the command line to be included

in the assembly.  These options are the following:

l	enable output listing.

nol	disable output listing (default).

cre	generate cross reference table.

s	generate a symbol table.

c	enable cycle count.

noc	disable cycle count.

The command line looks like this :

as* file1 file2 ... [ - option1 option2 ...]

If this method of passing commands to the assembler is used rather

than the OPT pseudo op code, a space should separate the minus sign

from the last file name and the first option.  Example:

as5 program -l cre >program.lst

This command assembles file 'program' with an output listing and a

cross reference table to the file 'program.lst'.

The `S1' formatted object file is placed in file `filename.S19', the

listing and error messages are written to  the standard output.  If

multiple files are assembled, the 'S1' file will be placed under the

first file's name.S19.

The listing file contains the address and bytes assembled for each

line of input followed by the original input line (unchanged, but

moved over to the right some). If an input line causes more than 6 bytes

to be output (e.g. a long FCC directive), additional bytes (up

to 64) are listed on succeeding lines with no address preceding

them.

Equates cause the value of the expression to replace the address

field in the listing.

Equates that have forward references cause Phasing Errors in Pass 2.

Expressions may consist of symbols, constants or the character '*'

(denoting the current value of the program counter) joined together

by one of the operators: +-*/%&|^. The operators are the same as in

C:

+	add

-	subtract

*	multiply

/	divide

%	remainder after division

&	bitwise and

|	bitwise or

^	bitwise exclusive-or

Expressions are evaluated left to right and there is no provision

for parenthesized expressions. Arithmetic is carried out in signed

twos-complement integer precision (16 bits on the IBM PC)

Constants are constructed with the same syntax as the Motorola MDOS

assembler:

'		followed by ASCII character

$		followed by hexadecimal constant

@		followed by octal constant

%		followed by binary constant

digit	decimal constant

ERRORS

Error diagnostics are placed in the listing file just before the

line containing the error.  Format of the error line is:

Line_number: Description of error

or

Line_number: Warning --- Description of error

Errors of the first type in pass one  cause  cancellation  of  pass

two.  Warnings	do  not  cause cancellation of pass two but should

cause you to wonder where they came from.

Error messages are meant to be self-explanatory.

If more than one file is being assembled, the  file name  precedes

the error:

File_name,Line_number: Description of error

Finally, some errors are classed as fatal and  cause an immediate

termination  of  the assembly.	Generally these errors occur when

a temporary file cannot be created or is lost  during  the assembly.

Consult your local guru if this happens.

DIFFERENCES

For indexed addressing, the comma is required before the  register;

`inc x' and `inc ,x' are not the same.

Macros are not supported.  (try M4 or M6)

The force size operators ('>' and  '<')  are implemented  for  all

assemblers.

The only pseudo-ops supported are:

ORG, FCC, FDB, FCB, EQU, RMB, BSZ, ZMB, FILL, PAGE and 	OPT.

The OPT pseudo-op allows the following operands:

nol		Turn off output listing

l		Turn on output listing (default)

noc		Disable cycle counts in listing (default)

c		Enable cycle counts in listing (clear total

cycles)

contc  		Re-enable cycle counts (don't clear total

cycles)

cre		Enable printing of a cross reference table

s		Generate a symbol table

Some of the more common pseudo-ops are not present:

SPC		Use blank lines instead

END		The assembly ends when there is no more

input

TTL		Use `pr' to get headings and page numbers

NAM[E]	Did you ever use this one anyway?

The above 4 pseudo-ops are recognized, but ignored.

ZMB		(Zero Memory Bytes) is equivalent to BSZ

(Block Store  Zeroes).

FILL can be used to initialize memory to something

other than zero: FILL val,nbytes.

TARGET MACHINE SPECIFICS

(as0)	6800:	Use for 6802 and 6808 too.

(as1)	6801:	You could use this one for  the  6800  and

avoid LSRD, ASLD, PULX, ABX, PSHX, MUL,

SUBD, ADDD, LDD and STD.

(as4)	6804:	The symbols 'a', 'x' and 'y'  are predefined

as $FF, $80 and $81 respectively.Also defined

as 'A',   'X' and   'Y'. Because  of  the  6804

architecture, this means that 'clr x'  will work

since  the  x register is just a memory

location. To  use  short-direct addressing,

the   symbol involved  must not be a forward

reference (i.e. undefined) and must be in the

range $80-$83. Remember  that	bytes assembled in

the range $10-$7F  will go into the data space;

There is no program space ROM for these locations.

The syntax for Register indirect addressing is

as follows:

mnemonic [<x>or<y>]

an example is:

lda [x]  the comma ',' is not allowed.

The MVI instruction (move immediate) has its own

format:  mvi address,#data where address is

an 8-bit address in page zero, and data is

the value to be written to specified location.

(as5)	6805:	There is no 'opt cmos' pseudo, so be careful not

to use STOP or WAIT in a program that is

destined for  an  NMOS version of the 6805. The

MUL instruction   should   also  be  avoided on

all versions of the 6805 except the C4. Cycle

times are for the NMOS versions.

(as9)	6809:	The SETDP pseudo-op is not implemented. Use the

'>' and '<` operators to force the size of

operands. For compatibility, CPX is equal to CMPX.

(as11)	68HC11:	Bit manipulation operands are separated by

blanks  instead of commas since the 'HC11 has

bit manipulation instructions that operate on

indexed addresses.

DETAILS

Symbol:	A string of  characters  with	a  non-initial

digit.  The string of characters may be from the

set: [a-z][A-Z]_.[0-9]$ ( . and _ count as non-

digits ).  The `$' counts as a digit to   avoid

confusion with  hexadecimal  constants.	All

characters of a symbol are  significant,  with

upper and lower  case  characters being

distinct.  The maximum number of characters in a

symbol is currently set at 15.

The symbol table has room for  at  least  2000

symbols  of length 8 characters or less.

Label:	A symbol starting in the first column is a label

and may optionally  be	ended  with a ':'. A

label may appear on a line by itself and is then

interpreted as:  Label	EQU	*

Mnemonic: A symbol preceded by at least one whitespace

character. Upper  case characters in this field

are converted to lower case before being checked

as a legal mnemonic. Thus `nop', `NOP' and even

`NoP' are recognized as the same mnemonic.

Note that register names that sometimes appear

at  the	end of  a  mnemonic (e.g. nega or stu)

must not be separated by any  whitespace

characters. Thus  `clra'   means   clear

accumulator A, but that `clr a' means clear

memory location `a'.

Operand: Follows mnemonic, separated by at  least  one

whitespace character.   The   contents   of the

operand  field  is interpreted by each

instruction.

Whitespace: A blank or a tab

Comment: Any text after all operands for  a  given

mnemonic  have been  processed or, a line

beginning with '*' up to the end of line or, an

empty line.

Continuations: If a line ends with a backslash (\) then

the  next line  is  fetched  and added to the

end of the first line.  This continues until a

line is seen which doesn't end in  \ or  until

MAXBUF characters have been collected (MAXBUF

>=256 ).

FILES

filename.S19	S-record output file

STDOUT		listing and errors (use redirection for

listing file)

Fwd_refs	Temporary file for forward references.

IMPLEMENTATION NOTES

This is a classic 2-pass assembler.  Pass 1 establishes the symbol

table and pass 2 generates the code.             12/11/84 E.J.Rupp

This version of the cross assemblers ported to the IBM PC 4/13/87